ESP32 Ethernet Tester

Portable Network Validation & Security Assessment Platform

A pocket-sized bench tool for stress-testing, validating, and security-auditing Ethernet switch ports. Full raw-frame control, protocol simulation, and offensive testing in a single PoE-powered device.

## Layer 1/2 Testing

Raw Ethernet frame TX/RX (W5500 MACRAW mode)

Error injection: giant, jumbo, broadcast, multicast, PAUSE, pattern storms

Continuous stress generation at configurable rates

PHY speed/duplex control (auto, 100FD, 100HD, 10FD, 10HD)

RFC 2544 benchmarks: throughput, latency, frame-loss, back-to-back

## Network Discovery & Reconnaissance

LLDP / CDP passive decode and active advertisement

Passive host/protocol mapping

ICMP ping sweep and traceroute

ARP scan (L2 host discovery)

TCP SYN port scan with banner grab (30+ service probes)

SNMP community probe, IP sweep, and write-access test

Combined network assessment with CIDR support

## Protocol Testing

Full DHCP DORA verification + 6 failure/abuse scenarios

DHCPv6 stateful server discovery (SOLICIT)

DNS A-record resolution and rogue DNS responder

mDNS .local resolution and reachability probe

IPv6 NDP decode (RS/RA/NS/NA)

FHRP (HSRP/VRRP) passive decode

## Security Assessment

802.1X supplicant: EAP-MD5, EAP-TLS, PEAPv0/MSCHAPv2, EAP-TTLS

Rogue authenticator with credential harvesting (hashcat output)

EAPOL-Start flood, spoofed logoff, MAB probe

L2 attacks: VLAN inject, Q-in-Q hop, DTP spoof, CAM flood, STP root claim

ARP MITM, gratuitous ARP, ARP storm

Rogue Router Advertisement (SLAAC takeover)

HSRP/VRRP gateway hijack

Rogue DHCP/DHCPv6 server, DNS spoofing

MAC randomization per-operation and per-host

## Management & Automation

Wi-Fi web UI: 8-tab interface with real-time output streaming

Serial CLI with 100+ commands

Scripting engine: loops, conditionals, delays, cron scheduling

PCAP capture to SD card (downloadable via web)

Output logging with timestamped log files

OTA firmware updates (TFTP + HTTP upload)

WireGuard VPN tunnel for secure remote management

HTTPS with basic authentication

NVS persistence for all configuration

## Hardware Platform

| MCU | ESP32-S3 (dual-core 240 MHz, 16 MB flash, 8 MB PSRAM) |
| --- | --- |
| Ethernet | WIZnet W5500 (onboard, PoE-capable) |
| Form Factor | Waveshare ESP32-S3-POE-ETH (compact single-board) |
| Power | USB-C or 802.3af PoE |
| Storage | Micro-SD card (FAT32) + LittleFS internal |

## Key Differentiators

True raw-frame control -- crafts any Ethernet frame at the MAC layer

All-in-one: L1 stress testing through L7 service identification in one device

No laptop required -- standalone operation via Wi-Fi web UI or scripted automation

Pocket-sized and PoE-powered -- deploy anywhere with a single cable

Arm/disarm safety model -- offensive tests locked behind explicit authorization

Open firmware with OTA updates -- continuously expanding capability set

## Use Cases

Switch Port Validation -- Verify port security, storm control, BPDU guard, DHCP snooping, and 802.1X enforcement on new deployments.

Network Penetration Testing -- Assess L2/L3 attack surface: VLAN hopping, ARP poisoning, rogue services, credential harvesting.

Troubleshooting -- LLDP/CDP neighbor discovery, DHCP diagnostics, DNS resolution testing, link-layer analysis.

Performance Benchmarking -- RFC 2544 throughput/latency/loss measurements for acceptance testing.

Compliance Auditing -- Verify 802.1X enforcement, SNMP hardening, and protocol filtering policies.