# ESP32 Ethernet Tester

A bench tool for stress-testing and validating Ethernet switch ports (built and tuned against an Aruba CX6300). An ESP32-S3 drives a WIZnet W5500 in raw MAC (MACRAW) mode, giving full control over the frames placed on the wire. A minimal IPv4 stack layered on top adds L3/DHCP testing, and a Wi-Fi web interface provides remote control with a tabbed multi-panel UI.

Hardware note (FCS): In MACRAW mode the W5500 auto-calculates and appends the 4-byte FCS, and auto-pads short frames to the 60-byte minimum. As a result, true CRC error injection and true runt injection are not possible through this hardware. See Limitations.

## Contents

Features

Hardware

Wiring

Build & Flash

Quick-Start Tutorial

Serial CLI Reference

Wi-Fi & Web Interface

L3 / DHCP Testing

802.1X / EAP Authentication

LLDP / CDP

FHRP (HSRP / VRRP)

DHCPv6

DNS Tools

SNMP Recon

Port Scanning & Banner Grab

Network Assessment

MAC Randomization

Rogue Authenticator

OTA Firmware Updates

RFC 2544 Suite

SD Card & File Manager

WireGuard VPN

Scripting & Automation

Architecture

Configuration (NVS)

Limitations

Troubleshooting

## Features

| Area | Capability |
| --- | --- |
| Raw frame TX/RX | Arbitrary Ethernet frames via W5500 MACRAW (socket 0, no MAC filter) |
| Error injection | Giant, jumbo, bad-EtherType, broadcast, multicast, PAUSE, payload patterns, broadcast storm |
| Continuous mode | Sustained error stream of any injectable type at a configurable rate |
| RFC 2544 | Throughput, latency, frame-loss, back-to-back (requires DUT loopback) |
| LLDP / CDP | Passive neighbour discovery (decode) and configurable advertisement (TX) |
| L3 / IP | Minimal ARP/IP/UDP/ICMP stack; ping; static or DHCP addressing |
| DHCP testing | Full DORA verification plus six failure/abuse scenarios + rogue server |
| DHCPv6 | Stateful IPv6 server discovery (SOLICIT) and rogue DHCPv6 server |
| DNS | Unicast DNS A-record resolution and rogue DNS responder (spoof) |
| mDNS probe | Resolve <host>.local over multicast DNS and ping for reachability |
| 802.1X / EAP | Full supplicant: EAP-MD5, EAP-TLS, PEAPv0/MSCHAPv2, EAP-TTLS (PAP + MSCHAPv2); configurable target MAC/IP |
| Rogue authenticator | Fake 802.1X authenticator to harvest EAP-MD5 / MSCHAPv2 credentials |
| EAPOL attacks | Start-flood, spoofed logoff, MAB probe |
| IPv6 / NDP | Passive NDP decode (RS/RA/NS/NA) and rogue Router Advertisement (SLAAC takeover) |
| FHRP (HSRP/VRRP) | Passive decode of HSRP/VRRP advertisements + gateway hijack |
| L2 attacks | 802.1Q VLAN inject, Q-in-Q hop, DTP spoof, CAM flood, STP root/TCN, LLDP/CDP flood |
| ARP tools | Gratuitous ARP, ARP MITM spoof, ARP storm, ARP scan |
| SNMP recon | Community probe, IP sweep, write-access test (sysContact.0 read/set/restore) |
| Port scanning | TCP SYN scan with banner grab for 30+ services, randomizable port order |
| Network assessment | Combined ping sweep + ARP scan + port scan + SNMP with CIDR, host/port randomization, per-host MAC rotation |
| Reconnaissance | Passive host/protocol mapping, ICMP ping sweep, traceroute |
| MAC randomization | Per-operation or per-host random locally-administered MACs; global auto-randomize option |
| OTA updates | Firmware update via TFTP (CLI) or HTTP file upload (web UI) |
| PCAP capture | Capture frames to SD card or LittleFS, downloadable from web UI |
| SD card | FAT32 file manager: ls, cat, rm, rename, mkdir, write; PCAP/log storage |
| WireGuard VPN | Tunnel management interface back to a central server |
| Scripting | Run multi-command scripts from SD with delays, loops, and conditionals |
| Cron scheduler | Time-based scheduled command execution (standard cron syntax) |
| Output logging | Tee all CLI output to timestamped log files on SD |
| File upload | HTTP POST upload of logs, pcaps, and arbitrary SD files to a server |
| Web server | HTTPS support, basic-auth, on/off control |
| Wi-Fi web UI | Tabbed multi-panel interface with real-time output; STA + AP modes |
| PHY control | Force link speed/duplex: auto, 100FD, 100HD, 10FD, 10HD |
| Persistence | Wi-Fi credentials, hostname, IP config, 802.1X creds, WireGuard keys stored in NVS |

## Hardware

| Item | Detail |
| --- | --- |
| MCU | Waveshare ESP32-S3-POE-ETH (ESP32-S3, USB-C, 16 MB flash, 8 MB PSRAM) |
| Ethernet | Onboard WIZnet W5500 (PoE-capable carrier) |
| Interface | FSPI @ 8 MHz (see note below) |

Ethernet, W5500 power, and (optional) PoE are all onboard — no external wiring is required. The SPI clock is set to 8 MHz in include/config.h; it can be raised toward 40 MHz since the W5500 is connected over short onboard PCB traces.

## Wiring

The W5500 is integrated on the Waveshare ESP32-S3-POE-ETH board, so no external Ethernet wiring is needed. The onboard SPI mapping (defined in include/config.h) is:

| W5500 signal | ESP32-S3 GPIO | Notes |
| --- | --- | --- |
| MOSI | 11 | FSPI MOSI |
| MISO | 12 | FSPI MISO |
| SCLK | 13 | FSPI SCK |
| CS | 14 | Chip select |
| RST | 9 | Hardware reset |
| INT | 10 | Not used in polling mode |

## Build & Flash

This is a PlatformIO project (platformio.ini, board = esp32-s3-devkitc-1, Arduino framework).

# Build
pio run

# Build, upload, and open the serial monitor (115200 baud)
pio run --target upload
pio device monitor -b 115200

Library dependencies (fetched automatically): mathieucarbou/ESPAsyncWebServer.

On boot the device prints W5500 init status, link state, attempts Wi-Fi (if enabled), and presents the ETH> prompt.

## Quick-Start Tutorial

This walks through a typical first session: boot, get an IP, run some tests, and use the web UI.

### 1. Connect and power on

Plug an Ethernet cable from the Waveshare board to a switch port. Connect USB-C for serial and power. Open a terminal at 115200 baud:

pio device monitor -b 115200

You should see W5500 init output and Link: UP.

### 2. Get an IP address

ETH> ip dhcp

The device performs a full DHCP DORA exchange. Verify with:

ETH> status

If DHCP is unavailable, assign a static address:

ETH> ip static 192.168.1.100 255.255.255.0 192.168.1.1

Commands that require an IP address (scan, recon sweep/trace, dns, snmp, probe, arp, fhrp hijack) will refuse to run and print a warning if no IP is configured.

### 3. Basic discovery

ETH> discover 30          # Listen for LLDP/CDP from the switch
ETH> recon passive 20     # Map hosts and protocols on the wire
ETH> arp scan 192.168.1.1 192.168.1.254  # ARP sweep for live hosts

### 4. Port scanning

ETH> scan common 192.168.1.1       # Scan common ports on the gateway
ETH> scan banner 192.168.1.1 22    # Grab SSH banner

### 5. Error injection and RFC 2544

ETH> inject giant 10               # Send 10 giant frames
ETH> inject storm 1000             # Start broadcast storm at 1000 fps
ETH> inject stop                   # Stop all injection

# For RFC 2544, enable port loopback on the DUT first:
ETH> test all                      # Full suite (throughput, latency, loss, burst)

### 6. Offensive tests (authorized networks only)

ETH> arm on                        # Enable offensive mode
ETH> l2 macflood 5000 10000        # CAM table flood
ETH> arp spoof 192.168.1.50 192.168.1.1 30   # ARP MITM for 30s
ETH> disarm                        # Lock out offensive tests

### 7. Enable the web UI

ETH> wifi ssid MyNetwork
ETH> wifi pass MyPassword
ETH> wifi on
ETH> reboot

After reboot, browse to http://esp32-tester.local/. The tabbed web UI gives access to all tools:

| Tab | Contents |
| --- | --- |
| Status | Live device status, SD card info |
| L1/L2 | PHY speed control, traffic generation, loopback |
| Inject | Error injection controls (one-shot and continuous) |
| Tests | RFC 2544, DHCP scenarios, DNS, offensive tests (when armed) |
| Recon | IP config, LLDP/CDP, network scan, port scanner, listeners |
| Security | 802.1X/EAP authentication |
| Files | PCAP capture, SD file browser, log viewer |
| Scripts | Script engine, cron scheduler, inline script runner |

The gear icon in the header opens the Settings modal for system controls, offensive mode arm/disarm, web server HTTPS, authentication, Wi-Fi, WireGuard, and upload destination configuration.

### 8. Scripting (automation)

Create a script on the SD card (via the web file manager or sd write):

ETH> sd write /scripts/daily.txt "# Daily port health check"
ETH> sd write /scripts/daily.txt "ip dhcp"
ETH> sd write /scripts/daily.txt "delay 2000"
ETH> sd write /scripts/daily.txt "discover 10"
ETH> sd write /scripts/daily.txt "recon passive 15"
ETH> sd write /scripts/daily.txt "arp scan 192.168.1.1 192.168.1.254"

Run it:

ETH> script run /scripts/daily.txt health-check

Output is automatically logged to /logs/health-check-YYYYMMDD-HHMMSS.log.

Schedule it with cron:

ETH> cron add 0 6 * * * script run /scripts/daily.txt morning

## Serial CLI Reference

Connect at 115200 baud. Type help (or ?) for the live list. Commands are case-insensitive.

### General

| Command | Description |  |
| --- | --- | --- |
| status | Link state, PHY info, MACs, modes |  |
| stats | TX/RX counters |  |
| stats clear | Reset counters |  |
| mac <XX:XX:XX:XX:XX:XX> | Set source MAC |  |
| target <XX:XX:XX:XX:XX:XX> | Set destination MAC |  |
| `loopback on\ | off` | Reflector mode (echo RX back to sender) |
| send <count> [size] | Send count test frames of size bytes |  |

### Error Injection

| Command | Description |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| inject runt [count] | Runt frames — see limitation; W5500 pads to 64 B so the DUT will *not* count these as runts |  |  |  |  |  |  |
| inject giant [count] | Giant frames (>1518 B on wire) |  |  |  |  |  |  |
| inject jumbo [count [size]] | Jumbo frames (default 9000 B; needs jumbo MTU enabled on the DUT) |  |  |  |  |  |  |
| inject broadcast [count] | Frames to FF:FF:FF:FF:FF:FF |  |  |  |  |  |  |
| inject multicast [count] | Frames to 01:00:5E:00:00:01 |  |  |  |  |  |  |
| inject badtype [count] | Reserved/undefined EtherType (0x88B6) |  |  |  |  |  |  |
| inject pause [count [quanta]] | 802.3x PAUSE frames |  |  |  |  |  |  |
| inject pattern <pat> [len] [n] | Stress pattern: `zeros\ | ones\ | alt\ | incr\ | random` |  |  |
| inject storm <rate_hz> | Background broadcast storm (0 to stop) |  |  |  |  |  |  |
| inject continuous <type> <rate_hz> [size] | Continuous error stream; type = `giant\ | jumbo\ | badtype\ | broadcast\ | multicast\ | pause\ | pattern` |
| inject continuous stop | Stop continuous injection |  |  |  |  |  |  |
| inject stop | Stop both storm and continuous injection |  |  |  |  |  |  |

### RFC 2544 (requires DUT port loopback)

| Command | Description |
| --- | --- |
| test throughput [size] | Zero-loss throughput (binary search) |
| test latency [size] | Round-trip latency |
| test frameloss [size] | Frame loss at 100/50/10 % load |
| test backtoback [size] | Max burst without loss |
| test all | Full RFC 2544 suite across all mandated frame sizes |

### Neighbour Discovery / Advertisement

| Command | Description |  |
| --- | --- | --- |
| discover [seconds] | Listen for and decode LLDP/CDP neighbours (default 65 s) |  |
| advertise | Show advertisement config |  |
| `advertise lldp on\ | off` | Toggle LLDP advertisement |
| `advertise cdp on\ | off` | Toggle CDP advertisement |
| advertise name <string> | System name / device ID |  |
| advertise port <string> | Port ID |  |
| advertise platform <string> | Platform / description string |  |
| advertise ip <a.b.c.d> | Management IPv4 (0.0.0.0 = none) |  |
| advertise vlan <id> | Port / native VLAN (0 = none) |  |
| advertise ttl <seconds> | Advertised TTL |  |
| advertise interval <seconds> | Transmit interval |  |
| advertise off | Disable both LLDP and CDP |  |

### L3 / IP Testing

| Command | Description |
| --- | --- |
| ip show | Show IP stack address config |
| ip dhcp | Obtain address via DHCP (DORA) and apply it |
| ip static <ip> <mask> <gw> | Set and persist a static IPv4 address |
| dhcp discover | Full DORA exchange (verify a server) |
| dhcp detect | Server presence / server-down detection |
| dhcp flood [count] | Starvation / pool-exhaustion test (unique random MACs) |
| dhcp decline | OFFER then DECLINE (address-conflict simulation) |
| dhcp nak [ip] | Request a bogus IP, expect NAK |
| dhcp malformed | Send a malformed DISCOVER, expect the server to ignore it |
| dhcp renew [secs] | Short lease, then unicast renew |
| probe <hostname> | Resolve <name>.local via mDNS and ping it |
| dns resolve <host> [server] | Unicast DNS A-record query (uses gateway if no server given) |

### 802.1X / EAP (EAPOL Supplicant)

| Command | Description |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| dot1x | Show 802.1X method / credential status |  |  |  |  |
| `dot1x method md5\ | tls\ | peap\ | ttls-pap\ | ttls-mschap` | Select EAP method |
| dot1x probe | Detect whether the port enforces 802.1X |  |  |  |  |
| dot1x auth [user] [pass] | Authenticate with the selected method |  |  |  |  |
| dot1x user <name> | Set 802.1X identity (saved) |  |  |  |  |
| dot1x pass <password> | Set EAP password (saved) |  |  |  |  |
| dot1x keypass <pw> | Set EAP-TLS private-key passphrase (saved) |  |  |  |  |
| dot1x cert [clear ...] | Show / remove uploaded certificates |  |  |  |  |
| dot1x logoff | Send EAPOL-Logoff |  |  |  |  |

### Reconnaissance

| Command | Description |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| recon passive [secs] | Passively map hosts/protocols (default 30 s) |  |  |  |  |
| recon sweep <start> <end> | ICMP ping sweep of an address range |  |  |  |  |
| recon trace <ip> [maxhops] | ICMP traceroute to a target |  |  |  |  |
| arp scan <start> <end> | ARP sweep for live hosts (L2 discovery) |  |  |  |  |
| scan common <ip> | TCP SYN scan of common ports |  |  |  |  |
| scan ports <ip> <first> <last> | TCP SYN scan of a port range |  |  |  |  |
| scan banner <ip> <port> [probe] | Banner grab via full TCP handshake |  |  |  |  |
| link [monitor [secs]] | Link speed/duplex info or flap monitor |  |  |  |  |
| `link speed auto\ | 100fd\ | 100hd\ | 10fd\ | 10hd` | Force PHY speed/duplex |
| wifi scan | Scan Wi-Fi (rogue-AP / evil-twin recon) |  |  |  |  |
| ipv6 listen [secs] | Decode IPv6 NDP (RS/RA/NS/NA) |  |  |  |  |
| fhrp listen [secs] | Decode HSRP/VRRP advertisements |  |  |  |  |
| dhcpv6 probe [secs] | Discover DHCPv6 servers (SOLICIT) |  |  |  |  |
| snmp probe <ip> | SNMP community-string probe (tries public/private/community/admin/snmp/monitor) |  |  |  |  |
| snmp sweep <start> <end> [comm] | SNMP sweep of an IP range |  |  |  |  |
| pcap start [secs] [maxframes] | Capture frames to /capture.pcap (web download) |  |  |  |  |
| `pcap status \ | delete` | Show or remove the stored capture |  |  |  |

### Offensive / DoS Tests (require arm)

| Command | Description |
| --- | --- |
| arm [on] | Show or enable authorized (lab) mode |
| disarm | Disable authorized mode |
| l2 vlan <vid> [pcp] [count] | 802.1Q single-tag VLAN inject |
| l2 dtag <native> <target> [n] | Double-tag (Q-in-Q) VLAN hop |
| l2 dtp | DTP trunk-negotiation spoof |
| l2 macflood [count] [rate] | CAM-table flood (random MACs) |
| l2 stp listen [secs] | Decode STP/RSTP BPDUs |
| l2 stp root [prio] [secs] | Claim root bridge (BPDU/Root Guard test) |
| l2 stp tcn [count] | Inject topology-change BPDUs |
| l2 lldpflood [count] | LLDP neighbour-table flood |
| l2 cdpflood [count] | CDP neighbour-table flood |
| arp gratuitous <ip> [count] | Gratuitous ARP announce |
| arp spoof <victim> <gw> [secs] | ARP MITM (auto-restores on stop) |
| arp storm [count] [rate] | ARP request flood |
| ipv6 rogue [lifetime] [count] | Rogue Router Advertisement (SLAAC takeover) |
| fhrp hsrp <grp> <vip> [prio] [n] | HSRP gateway hijack |
| fhrp vrrp <vrid> <vip> [prio] [n] | VRRP master hijack |
| dhcpv6 rogue <prefix> <dns> [s] | Rogue DHCPv6 server (stateful IPv6) |
| dns spoof <ip> [secs] | DNS spoof (answer all queries with <ip>) |
| dhcp rogue <pool> [mask gw dns secs] | Rogue DHCP server (hands out leases) |
| dot1x startflood [count] | EAPOL-Start flood (random MACs) |
| dot1x logoffmac <mac> | Spoofed EAPOL-Logoff (deauth a client) |
| dot1x mab [secs] | MAB / 802.1X enforcement probe |
| dot1x rogue [secs] [md5] | Rogue authenticator (harvest EAP credentials) |

### SD Card / File Manager

| Command | Description |
| --- | --- |
| sd | Show TF/SD card info |
| sd init | Re-detect / remount the SD card |
| sd format | Erase and format SD card (FAT32) |
| sd ls [path] | List directory contents |
| sd cat <file> | Print file contents |
| sd rm <file> | Delete a file |
| sd rename <old> <new> | Rename / move a file |
| sd mkdir <path> | Create a directory |
| sd write <file> <text> | Append a line of text to a file |

### WireGuard VPN

| Command | Description |  |
| --- | --- | --- |
| wg | Show tunnel status and config |  |
| wg set localip <ip> | Set tunnel interface IP |  |
| wg set privkey <base64> | Set local private key |  |
| wg set pubkey <base64> | Set peer public key |  |
| `wg set endpoint <host\ | ip>` | Set peer endpoint address |
| wg set port <port> | Set peer endpoint port (default 51820) |  |
| wg set psk <base64> | Set pre-shared key (optional) |  |
| wg enable | Enable auto-start on boot |  |
| wg disable | Disable and stop tunnel |  |
| wg start | Start tunnel now |  |
| wg stop | Stop tunnel |  |
| wg clear | Erase all WireGuard config |  |

### Scripting & Automation

| Command | Description |
| --- | --- |
| script | Show script engine status |
| script list | List scripts on SD (/scripts/) |
| script run <file> [logname] | Run a script (optional auto-log) |
| script stop | Abort running script |
| script create <file> | Create a template script on SD |

### Output Logging

| Command | Description |
| --- | --- |
| log | Show logger status |
| log start [name] | Start logging to SD (auto-name if omitted) |
| log stop | Stop logging |
| log list | List log files on SD |
| log delete <file> | Delete a log file |
| log flush | Force flush to SD |

### Cron Scheduler

| Command | Description |
| --- | --- |
| cron | List scheduled tasks |
| cron add <min> <hr> <dom> <mon> <dow> <cmd> | Add a cron entry |
| cron remove <index> | Remove entry by index |
| cron reload | Re-read /cron.txt from SD |
| cron clear | Remove all entries |

### File Upload

| Command | Description |
| --- | --- |
| upload | Show upload destination config |
| upload set <url> [user] [pass] | Set default upload server |
| upload clear | Clear upload destination |
| upload log <name> [url] | Upload a log file (HTTP POST) |
| upload pcap [url] | Upload current pcap capture |
| upload file <path> [url] | Upload any SD file |

### Wi-Fi / Web (persisted to NVS)

| Command | Description |  |
| --- | --- | --- |
| wifi | Show Wi-Fi status |  |
| wifi ssid <ssid> | Set SSID |  |
| wifi pass <password> | Set password |  |
| `wifi mode ap\ | sta` | Mgmt as soft AP (field) or station |
| wifi apssid <ssid> | Set soft-AP SSID |  |
| wifi appass <password> | Set soft-AP password (>=8 chars, blank=open) |  |
| `wifi on\ | off` | Enable/disable Wi-Fi on boot |
| host <hostname> | Set device hostname (used by the mDNS responder) |  |
| web | Show web server status |  |
| `web on\ | off` | Start/stop web server |
| web auth set <user> <pass> | Set basic-auth credentials (saved) |  |
| web auth clear | Disable authentication |  |
| `web https on\ | off` | Enable/disable HTTPS (port 443) |

Wi-Fi/host changes are saved immediately but applied on the next reboot.

## Wi-Fi & Web Interface

Implemented in src/wifi_web.cpp.

STA auto-connect: if Wi-Fi is enabled and credentials exist in NVS, the

device connects as a station on boot and registers <hostname>.local via mDNS (ESPmDNS).

AP fallback: if no credentials exist or the connection fails, it starts

an open AP named `ESP32-Tester-Setup` hosting the same control page so credentials can be entered.

Soft-AP mode (`wifi mode ap`): the device always broadcasts its own AP

for field use without infrastructure Wi-Fi.

### Web UI

Browse to http://<hostname>.local/ (STA) or the AP IP (fallback). The interface uses a tabbed layout with eight functional tabs:

| Tab | Contents |
| --- | --- |
| Status | Auto-refreshing device status (link, speed, MAC, IP, counters, heap, SD), SD card init/format |
| L1/L2 | PHY speed selector, traffic generation (send frames), loopback toggle |
| Inject | One-shot and continuous error injection controls |
| Tests | RFC 2544 suite, DHCP test scenarios, DNS resolve, offensive tests (hidden until armed) |
| Recon | IP configuration (DHCP/static), LLDP/CDP discovery, network scan, port scanner, protocol listeners |
| Security | 802.1X/EAP authentication (method, credentials, auth) |
| Files | PCAP capture start/stop/download, SD card file browser, log file viewer |
| Scripts | Script engine (run/stop/list), cron scheduler, inline multi-line script runner |

A gear icon in the header opens the Settings modal with:

System info and reboot

Offensive mode arm/disarm (with confirmation dialog)

Web server on/off and HTTPS toggle

Basic authentication set/clear

Wi-Fi configuration (STA and AP mode)

WireGuard VPN full config

Upload destination

The UI auto-detects mobile/desktop layouts and includes an output panel that streams command results in real time.

### Security

Basic auth: web auth set <user> <pass> protects all endpoints (saved to

NVS, survives reboot).

HTTPS: web https on enables TLS on port 443 with an auto-generated

self-signed certificate.

Arm/disarm: offensive tests are hidden in the web UI and blocked in the

CLI until the device is explicitly armed.

### HTTP API

| Method | Path | Body | Purpose |
| --- | --- | --- | --- |
| GET | / | -- | Control page (HTML) |
| GET | /api/status | -- | JSON status object |
| POST | /api/cmd | c=<command> | Queue a CLI command for execution |
| GET | /api/result | -- | Poll command output and running state |
| POST | /api/config | ssid,pass,host,wifi | Save Wi-Fi config to NVS |
| GET | /api/files | -- | List SD card files (JSON) |
| GET | /api/download?f=<path> | -- | Download a file from SD |
| POST | /api/upload | multipart file | Upload a file to SD |

Commands received over HTTP are queued and executed from the main loop so the async server callbacks never block; output appears in the web output panel via polling.

## L3 / DHCP Testing

A deliberately minimal IPv4 stack (src/ip_stack.cpp) runs on top of MACRAW — the W5500 hardware TCP/IP stack is not used, preserving full raw-frame control. It implements:

ARP (request/reply + an 8-entry cache; answers ARP for our IP)

IPv4 (header build + Internet checksum)

UDP (send/receive with pseudo-header checksum)

ICMP (echo request/reply — ping, and replies to inbound pings)

Destination MAC resolution for broadcast, IPv4 multicast mapping, and on/off-subnet unicast (via ARP)

### DHCP scenarios

src/dhcp_test.cpp:

| Scenario | What it validates |
| --- | --- |
| discover (DORA) | Server hands out a lease correctly (OFFER → REQUEST → ACK) |
| detect | Whether *any* server answers (server-down detection) |
| flood | Pool exhaustion / starvation behaviour (unique client MAC per DISCOVER) |
| decline | Server marks an address conflicted after DECLINE |
| nak | Server correctly NAKs a request for an out-of-pool/bogus IP |
| malformed | Server ignores a packet with a bad magic cookie + truncated options |
| renew | Renew/rebind handling via a short lease + unicast renew |

## LLDP / CDP

src/discovery.cpp provides both directions:

Listen (`discover`) — decodes LLDP (EtherType 0x88CC) and CDP

(01:00:0C:CC:CC:CC, LLC/SNAP) PDUs the switch emits: system name, port, TTL, capabilities, management address, VLAN, platform, etc.

Advertise (`advertise`) — builds and periodically transmits configurable

LLDP and CDP PDUs. CDP uses proper LLC/SNAP encapsulation and Cisco's sign-extending checksum.

## 802.1X / EAP Authentication

src/dot1x.cpp implements a full 802.1X supplicant supporting multiple EAP methods:

| Method | Description |
| --- | --- |
| EAP-MD5 | Simple challenge/response (username + password) |
| EAP-TLS | Certificate-based mutual authentication (RFC 5216); client cert + key uploaded via web UI |
| PEAPv0/MSCHAPv2 | TLS tunnel with inner EAP-MSCHAPv2 (RFC 2759); most common enterprise Wi-Fi/wired auth |
| EAP-TTLS/PAP | TLS tunnel with inner PAP (cleartext inside tunnel, RFC 5281) |
| EAP-TTLS/MSCHAPv2 | TLS tunnel with inner MS-CHAPv2 via Diameter AVPs |

The TLS engine uses mbedTLS (ESP-IDF); fragmented EAP-TLS exchange is handled per RFC 5216. Certificates are stored in LittleFS and managed via the web UI or dot1x cert CLI commands.

Target specification: By default EAPOL frames are sent to the PAE multicast group (01:80:C2:00:00:03). Use dot1x target <MAC> to direct frames to a specific authenticator's unicast MAC, or dot1x target <IP> to record the authenticator's IP (useful for documentation/scripting). Clear with dot1x target clear.

## FHRP (HSRP / VRRP)

src/fhrp.cpp provides First-Hop Redundancy Protocol assessment:

Listen (`fhrp listen`) — passively sniffs and decodes HSRP (UDP/1985) and

VRRP (IP protocol 112) advertisements, revealing group IDs, virtual IPs, priorities, and state.

HSRP hijack (`fhrp hsrp`) — sends HSRP Coup + Hello frames with priority

255 using the HSRP virtual MAC (00:00:0c:07:ac:GG), taking over the default gateway. Tests HSRP authentication and FHRP hardening.

VRRP hijack (`fhrp vrrp`) — sends VRRP advertisements with max priority

using the VRRP virtual MAC (00:00:5e:00:01:VR), taking over the master role.

## DHCPv6

src/dhcpv6.cpp implements stateful IPv6 address assignment testing per RFC 8415:

Probe (`dhcpv6 probe`) — sends SOLICIT and decodes ADVERTISE/REPLY,

revealing the server DUID, offered IPv6 address, and DNS servers.

Rogue server (`dhcpv6 rogue`) — answers SOLICIT/REQUEST with

ADVERTISE/REPLY, handing out addresses from a specified /64 prefix and advertising a controlled DNS server. Tests DHCPv6 guard/snooping.

## DNS Tools

src/dns_tool.cpp provides DNS assessment capabilities:

Resolve (`dns resolve`) — unicast DNS A-record query to a specified server

(or the configured gateway). ARP-resolves the target, sends a standard DNS query, and parses the response.

Spoof (`dns spoof`) — rogue DNS responder that monitors the wire for DNS

queries (UDP/53) and injects forged replies redirecting all queried names to a controlled IP. Tests DNS inspection / DNSSEC enforcement.

## SNMP Recon

src/snmp_recon.cpp performs SNMP reconnaissance:

Probe (`snmp probe <ip> [community ...]`) — sends SNMPv1 GET requests for

sysDescr.0 (OID 1.3.6.1.2.1.1.1.0) using specified or default community strings (public, private, community, admin, snmp, monitor). Reports any accepted community and the device description.

Sweep (`snmp sweep <cidr|start> [end] [community]`) — probes an IP range

(up to 1024 hosts, supports CIDR notation) with a given community string, identifying all SNMP-responsive devices.

Write Test (`snmp writetest <ip> [write-community]`) — tests write access

by reading sysContact.0, setting it to a test marker, verifying the write, then restoring the original value. Confirms whether the community string has SET privileges.

The web UI provides separate read/write community inputs and dedicated Probe, Sweep, and Write Test buttons.

## Port Scanning & Banner Grab

src/portscan.cpp implements TCP SYN scanning with service identification:

Common scan (`scan common <ip>`) — scans well-known ports (21, 22, 23, 25,

53, 80, 110, 143, 443, 445, 993, 995, 3306, 3389, 5432, 8080, 8443)

Range scan (`scan ports <ip> <first> <last>`) — scans a contiguous port range

Banner grab (`scan banner <ip> <port>`) — connects and probes a single port

for service identification

Service probing supports 30+ protocol-specific handshakes including HTTP, SSH, FTP, Telnet, SMTP, POP3, IMAP, MySQL, PostgreSQL, Redis, MongoDB, MSSQL, RDP, VNC, SIP, LDAP, Elasticsearch, Docker, Kubernetes, RTSP, Memcached, and more. Unknown services receive a generic \r\n nudge to elicit a banner.

Port scan order can be randomized (Fisher-Yates shuffle) to evade sequential scan detection.

## Network Assessment

src/net_assess.cpp provides a combined multi-phase network assessment that runs four scan types in sequence:

Ping sweep — ICMP echo to all hosts in range

ARP scan — L2 discovery of live hosts

Port scan + banner grab — TCP SYN scan of specified ports (default:

21, 22, 80, 443) with service identification on open ports

SNMP discovery — probes all hosts with community string public

ETH> assess 192.168.1.0/24 22,80,443,8080 -r -m

Options:

IP range via CIDR notation or explicit start/end addresses

Custom port list (comma-separated)

-r — randomize host iteration and port scan order (Fisher-Yates)

-m — randomize source MAC per target host (locally-administered unicast,

original MAC restored after completion)

The web UI provides a Network Assessment card with CIDR/port inputs and checkboxes for randomization options.

## MAC Randomization

The tester supports source MAC randomization at multiple levels to reduce detectability during scanning operations:

Per-operation — "Random src MAC" checkboxes on Network Scan, Port Scanner,

SNMP Scan, and 802.1X web UI cards. When checked, a fresh locally-administered unicast MAC is generated before the operation.

Per-host — the Network Assessment -m flag generates a unique MAC for

each target host, then restores the original after the scan completes.

Global default — mac autorand on enables auto-randomization system-wide.

When enabled, all per-card checkboxes are pre-checked on page load. This is overridden when the user explicitly sets a MAC address via mac <XX:...>.

ETH> mac random            # generate a random MAC now
ETH> mac autorand on       # enable auto-randomize (persistent, default: off)
ETH> mac autorand off      # disable

## Rogue Authenticator

src/rogue_auth.cpp implements a fake 802.1X authenticator for credential harvesting (authorized pentest use):

Broadcasts EAP-Request/Identity to solicit supplicants (or responds to

EAPOL-Start frames)

Challenges with EAP-MD5 or EAP-MSCHAPv2 (configurable)

Captures and displays:

- Outer identity (often the real username) - EAP-MD5 challenge/response pairs - MS-CHAPv2 auth-challenge / peer-challenge / NT-Response

Outputs MS-CHAPv2 hashes in hashcat mode 5500 format for offline cracking

Tracks up to 8 concurrent supplicants

## OTA Firmware Updates

src/ota.cpp provides over-the-air firmware updates:

TFTP (ota tftp <server-ip> [filename]) — fetches a firmware binary from a

TFTP server (RFC 1350) and flashes it to the alternate OTA partition. Supports progress display and abort via serial/web.

HTTP upload (web UI) — direct file upload via the Settings modal. The

device accepts a .bin file via multipart POST to /api/ota, flashes it, and reboots automatically.

The build process generates ota/firmware.bin (and a versioned copy) via the ota_copy.py post-build script. The 16 MB flash uses a dual-OTA partition scheme (6.25 MB per slot).

## RFC 2544 Suite

src/rfc2544.cpp implements the four core benchmarks (throughput, latency, frame-loss, back-to-back) per RFC 2544. These require the DUT to reflect frames back to the tester. Two options:

Port loopback (one cable): interface 1/1/X → loopback on the CX6300.

Cable loop (two cables): connect port A to the tester and loop port B back

to port A.

Frame sizes and durations are configured in include/config.h (RFC2544_SIZES, RFC2544_TEST_SEC, etc.).

## SD Card & File Manager

The Waveshare ESP32-S3-POE-ETH has an onboard TF (micro-SD) card slot connected via a dedicated HSPI bus (separate from the W5500 SPI). Insert a FAT32-formatted micro-SD card for:

PCAP capture storage (larger captures than LittleFS allows)

Script storage (/scripts/*.txt)

Output logging (/logs/*.log)

Cron configuration (/cron.txt)

General file storage (certificates, configs, data exports)

The SD card is auto-detected on boot. Use sd init to remount after hot-insert, or sd format to erase and format.

| Pin | ESP32-S3 GPIO |
| --- | --- |
| CS | 4 |
| MOSI | 6 |
| MISO | 5 |
| SCK | 7 |

## WireGuard VPN

The device can establish a WireGuard tunnel over Wi-Fi for secure remote management. Configure via CLI or the web Settings modal:

ETH> wg set localip 10.0.0.2
ETH> wg set privkey <base64-private-key>
ETH> wg set pubkey <base64-peer-public-key>
ETH> wg set endpoint vpn.example.com
ETH> wg set port 51820
ETH> wg enable
ETH> reboot

When enabled, the tunnel starts automatically on boot after Wi-Fi connects. The wg set weboff on option stops the web server when the tunnel is active (security hardening for field deployments).

## Scripting & Automation

Scripts are plain-text files on the SD card (one CLI command per line). Special directives:

| Directive | Description |
| --- | --- |
| # comment | Ignored |
| delay <ms> | Pause execution for N milliseconds |
| repeat <n> | Repeat the next command N times |
| loop <n> ... endloop | Loop a block of commands N times |
| if <condition> ... endif | Conditional execution |
| log start [name] | Start output logging within script |
| log stop | Stop logging |

The cron scheduler reads /cron.txt from the SD card on boot (standard 5-field cron format) and executes commands at the scheduled times. The device must have NTP time (obtained automatically via Wi-Fi) for cron to function.

## Architecture

┌─────────────────────────────────────────────┐
                       │                  main.cpp                    │
                       │  setup(): init, NVS load, Wi-Fi, web         │
                       │  loop():  CLI · inj.tick · loopback ·         │
                       │           advertise · web                    │
                       └───────────────┬─────────────────────────────┘
                                       │
        ┌───────────┬───────────┬──────┴──────┬────────────┬───────────┐
        ▼           ▼           ▼             ▼            ▼           ▼
   ┌─────────┐ ┌─────────┐ ┌──────────┐ ┌──────────┐ ┌─────────┐ ┌──────────┐
   │   CLI   │ │ Error   │ │ RFC2544  │ │ Discovery│ │ IpStack │ │WebControl│
   │ (cli)   │ │ Inject  │ │          │ │ LLDP/CDP │ │ ARP/IP/ │ │ WiFi +   │
   │         │ │         │ │          │ │          │ │ UDP/ICMP│ │ Async UI │
   └────┬────┘ └────┬────┘ └────┬─────┘ └────┬─────┘ └────┬────┘ └────┬─────┘
        │           │           │            │            │           │
        │           │           │            │       ┌────▼─────┐     │
        │           │           │            │       │ DhcpTest │     │
        │           │           │            │       │ NetProbe │     │
        │           │           │            │       └────┬─────┘     │
        └───────────┴───────────┴────────────┴────────────┘           │
                                 │                                     │
                          ┌──────▼───────┐                    ┌────────▼────────┐
                          │   W5500Raw   │                    │   NetConfig     │
                          │ MACRAW / SPI │                    │   (NVS / Prefs) │
                          └──────────────┘                    └─────────────────┘

| File | Responsibility |
| --- | --- |
| [src/main.cpp](src/main.cpp) | Wiring, boot sequence, main loop |
| [src/w5500_raw.cpp](src/w5500_raw.cpp) | Low-level W5500 MACRAW SPI driver (socket 0) |
| [src/packet.cpp](src/packet.cpp) | Ethernet frame builders, patterns, MAC helpers |
| [src/error_inject.cpp](src/error_inject.cpp) | One-shot, storm, and continuous injectors |
| [src/rfc2544.cpp](src/rfc2544.cpp) | RFC 2544 benchmark suite |
| [src/discovery.cpp](src/discovery.cpp) | LLDP/CDP decode + advertisement |
| [src/ip_stack.cpp](src/ip_stack.cpp) | Minimal ARP/IP/UDP/ICMP over MACRAW |
| [src/dhcp_test.cpp](src/dhcp_test.cpp) | DHCP DORA + failure scenarios + rogue server |
| [src/dhcpv6.cpp](src/dhcpv6.cpp) | DHCPv6 client probe + rogue server (RFC 8415) |
| [src/dns_tool.cpp](src/dns_tool.cpp) | Unicast DNS resolver + rogue DNS responder |
| [src/snmp_recon.cpp](src/snmp_recon.cpp) | SNMPv1 community probe + sweep + write test |
| [src/net_probe.cpp](src/net_probe.cpp) | mDNS resolve + ping reachability |
| [src/dot1x.cpp](src/dot1x.cpp) | 802.1X supplicant (MD5/TLS/PEAP/TTLS) + EAPOL attacks |
| [src/rogue_auth.cpp](src/rogue_auth.cpp) | Rogue 802.1X authenticator / EAP credential harvester |
| [src/mschapv2.cpp](src/mschapv2.cpp) | MS-CHAPv2 crypto (self-contained MD4 + DES) |
| [src/cert_store.cpp](src/cert_store.cpp) | LittleFS certificate storage for EAP-TLS |
| [src/l2_attack.cpp](src/l2_attack.cpp) | L2 attack suite (VLAN/DTP/STP/MAC-flood/LLDP-CDP flood) |
| [src/arp_tool.cpp](src/arp_tool.cpp) | ARP scan, gratuitous, spoof, storm |
| [src/fhrp.cpp](src/fhrp.cpp) | FHRP assessment (HSRP/VRRP listen + hijack) |
| [src/ipv6_tool.cpp](src/ipv6_tool.cpp) | IPv6 NDP listen + rogue RA |
| [src/portscan.cpp](src/portscan.cpp) | TCP SYN scan + banner grab (30+ services) |
| [src/recon.cpp](src/recon.cpp) | Passive recon, ping sweep, traceroute |
| [src/net_assess.cpp](src/net_assess.cpp) | Combined network assessment (ping+ARP+ports+SNMP) |
| [src/ota.cpp](src/ota.cpp) | OTA firmware updates (TFTP + HTTP stream) |
| [src/linkdiag.cpp](src/linkdiag.cpp) | Link diagnostics + flap monitor |
| [src/pcap.cpp](src/pcap.cpp) | PCAP capture to SD card or LittleFS |
| [src/net_config.cpp](src/net_config.cpp) | NVS persistence (Preferences) |
| [src/wifi_web.cpp](src/wifi_web.cpp) | Wi-Fi STA/AP + ESPAsyncWebServer tabbed UI |
| [src/weblog.cpp](src/weblog.cpp) | TeeStream ring buffer for web output capture |
| [src/cli.cpp](src/cli.cpp) | Serial command parser and dispatch |
| [include/config.h](include/config.h) | Pins, constants, defaults |

## Configuration (NVS)

Persisted via the Preferences library in the `tester` namespace (src/net_config.cpp):

| Key | Field | Default |
| --- | --- | --- |
| ssid | Wi-Fi SSID | (empty) |
| pass | Wi-Fi password | (empty) |
| host | Hostname (<host>.local) | esp32-tester |
| wifiEn | Connect Wi-Fi on boot | false |
| wifiMode | Wi-Fi mode (0=STA, 1=AP) | 0 |
| apSsid | Soft-AP SSID | ESP32-Tester |
| apPass | Soft-AP password | (empty/open) |
| dhcp | Use DHCP for the Ethernet IP | true |
| ip / mask / gw | Static IPv4 config | 0 / 255.255.255.0 / 0 |
| webUser / webPass | Basic-auth credentials | (empty/disabled) |
| httpsOn | HTTPS enabled | false |
| armed | Offensive mode state | false |
| wgLocalIp | WireGuard tunnel IP | (empty) |
| wgPrivKey | WireGuard private key | (empty) |
| wgPubKey | WireGuard peer public key | (empty) |
| wgEndpoint | WireGuard peer endpoint | (empty) |
| wgPort | WireGuard peer port | 51820 |
| wgEnabled | WireGuard auto-start | false |
| uploadUrl | Default upload server URL | (empty) |
| d1xuser | 802.1X identity | (empty) |
| d1xpass | 802.1X password | (empty) |
| d1xmeth | EAP method (0=MD5, 1=TLS, 2=PEAP, 3=TTLS/PAP, 4=TTLS/MSCHAP) | 0 |
| d1xkeypw | EAP-TLS private key passphrase | (empty) |
| d1xtgt | 802.1X target authenticator MAC | (all-zeros/PAE mcast) |
| d1xtgtip | 802.1X target IP | 0 |
| rmacdf | Auto-randomize source MAC | false |

## Limitations

These stem from the W5500 hardware operating in MACRAW mode:

No CRC/FCS error injection. The W5500 always computes and appends a valid

FCS; corrupted-CRC frames require a hardware bit manipulator.

No true runts. Frames shorter than 60 bytes are auto-padded to the

60-byte minimum before FCS, so they leave the wire at the legal 64-byte minimum. The inject runt command is retained but prints a warning and the DUT will not count the frames as runts.

Jumbo frames require jumbo MTU to be enabled on the DUT port; otherwise

they are dropped or counted as giants.

The minimal IP stack is intentionally lightweight (no fragmentation, single

outstanding request per exchange) — sufficient for testing, not a general purpose stack.

## Troubleshooting

### W5500 init fails -- Version check failed

The driver reads the W5500 version register (expected 0x04). The value read is diagnostic:

| Read value | Likely cause |
| --- | --- |
| 0x7F or random | SPI returning garbage -- MISO line issue or clock too fast. |
| 0x00 | MISO delivering no data -- disconnected or no 3.3 V power. |
| 0xFF | MISO stuck high / module unpowered or absent. |

On the Waveshare ESP32-S3-POE-ETH, SPI is onboard -- if init fails, check for a damaged board or firmware misconfiguration.

### Web UI not reachable

Check wifi shows connected (serial prints the IP on boot).

Try the IP directly if <hostname>.local mDNS resolution fails.

Verify web shows the server is running (web on to start).

If basic auth is set, the browser will prompt for credentials.

In AP mode, join the AP SSID and browse to the AP IP printed on console.

### Commands refuse to run -- "No IP address configured"

Commands that require network connectivity (scan, recon sweep/trace, dns, snmp, probe, arp scan/spoof/storm, fhrp hijack) require an IP address. Run:

ETH> ip dhcp

or assign a static IP before using these tools.

### DHCP / probe commands do nothing useful

These depend on the W5500 being initialised and the link being up. Resolve any W5500 init failure first, then confirm status shows Link: UP.